RFID Top Ceiling Antenna

Product introduction:

The ceiling access control antenna is a newly developed high-performance UHF intelligent ceiling-mounted access control. It supports ISO18000-6C protocol, with an optional output power of 10dBm to 30dBm, strong multi-tag recognition capability, strong anti-interference ability, good scalability, and a stylish appearance. It is particularly suitable for shopping malls, clothing stores, shoe and hat stores, and luggage stores.

Characteristics:

- Independent intellectual property design

- Support EPC C1 G2 (ISO18000-6C) protocol

- Output power 10dBm-33dBm adjustable

- Strong multi-tag reading capability, with high-speed tag reading performance

- Simple operation, stylish appearance, easy assembly

- Built-in alarm light and buzzer, sensitive alarm, safe and reliable

Technical Parameters:

| Hardware performance |  |  |
| --- | --- | --- |
| Mainboard | RDIOT_T101_V1.01 |  |
| Hardware interface |  |  |
| Communication interface | RJ45(10M/100M),RS232（9600-230400bps）,RS485（19200-230400bps） |  |
| Optional | WIFI(2.4G), 4G ,POE（802.3af/802.3at） |  |
| Protocol | Modbus, HID, HTTP, HTTPS, MQTT, TCP |  |
| Instruction lights | Power on（Blue）,alarm（red） |  |
| GPIO | 4 inputs, 8 strong drive outputs (each channel can pull current up to 250mA) |  |
| Power supply/power consumption |  |  |
| Power supply | Power supply 9-24V; Standard adapter 12V/2A; POE power supply (compatible with 802.3af or 802.3at standards) |  |
| Consumption |  | Standby 1.76W; operating: 16.2W |
|  | Single board | operating 0.7W |
| POE power supply capacity | 100m Category 5e network cable, different models of POE switches have deviations Use 803.at power supply, maximum load capacity 25.1w (11.93V/2.1A) |  |
| Environment requests |  |  |
| Operate temperature | -25℃ - +65℃ |  |
| Stock temperature | -45℃ - +85℃ |  |
| Humidity | Relative humidity：5-95% |  |
| Safety |  |  |
| Power adapts | Air discharge 8KV, contact discharge 6KV; surge immunity: 4KV |  |
| Single board | Contact discharge:8KV |  |
| Dimension |  |  |
| Size | 200mm X 575mm X 45mm |  |

Outlook dimension

Installations

Cable connection

- Connect power cable with adapter

- connect power adapter with antenna power port;

- Connect the network cable to the network cable port of the ceiling antenna and connect the other end to the computer.

- Connect power plug, then system works；

Test lightings:

Antennas works with blue light, and alarm with red light.

（alarming）

（power on）